From: ballyalley@yahoogroups.com <ballyalley@yahoogroups.com> on behalf of maginnovision@juno.com [ballyalley] <ballyalley@yahoogroups.com>

Sent: Wednesday, January 16, 2019 10:40 PM

To: ballyalley@yahoogroups.com

Subject: Re: [ballyalley] Re: New Bally expansion board development [11 Attachments]

[Attachment(s) from maginnovision@juno.com [ballyalley] included below]

Ok so I've attached some images here. Original refers to the circuit implemented from the schematic on ballyalley(can't find the link right now), fixed refers to an altered version where the input cap has polarity reversed and increased to 47uF along with the 75 ohm series resistor right on the output. Finally the "new" labelled images are the newest circuit, which maybe surprisingly is the simplest considering the results are very good.

In the images... In original video.jpg you can see the output is somewhat fuzzy but colors actually aren't bad. In original shadows.jpg you can see some heavy shadows or ghosting as adam called it. This is due to the way colors are produced from a composite waveform which brings us to original video.PNG. The yellow waveform is at the modulator, as close to the source as possible. Orange is the output at the composite connector. You can see the tilt(the top edge of the wavform tilts downward) along the top marker. Even though the waveform exhibits some 60hz influence the tilt is still more excessive than the 60hz alone would cause. Notice also the Vpp for both waveforms is about 2V. Composite should be ~1Vpp nominally.

Fixed video... You can see is much sharper but still you can see some shadows. Fixed shadows you can more clearly see they're not much improved from original. Still heavy shadowing, and it's not the sharpest picture. In fixed video.png You can see we still have some 60hz influence but it is much reduced, the tilt is also quite a bit better, but not as nice as the yellow waveform. Our yellow waveform is still 2Vpp however the output is now fixed at ~1Vpp.

New video... You can see the image is sharp and colors are uniform. In new shadows you can see there are still some issues around the M and somewhat the G but still much improved. Looking at the wave from the Vpp for both waveforms are close to 1V, and there is no noticeable tilt. In addition the output waveform exhibits slightly less noise(more uniform tops and bottom) as well but noise is unavoidable due to how the color is added to the luma.

I've attached picture of the test circuit(Video bottom.jpg, Video Top.jpg) If you liked this data, great, if not you can still see why a board like this expansion board might take a while even if life weren't busy. I do hope someone got something out of this and I will post the schematic tomorrow for anyone interested enough to try it.

__._,_.___

Attachment(s) from maginnovision@juno.com [ballyalley] | View attachments on the web

10 of 11 Photo(s) (View all Photos)

Video bottom.jpg

Video bottom.jpg

fixed shadows.jpg

fixed shadows.jpg

fixed video.PNG

fixed video.PNG

new shadows.jpg

new shadows.jpg

New vid.jpg

New vid.jpg

original video.PNG

original video.PNG

fixed video.jpg

fixed video.jpg

original shadows.jpg

original shadows.jpg

original video.jpg

original video.jpg

new video.PNG

new video.PNG

Posted by: maginnovision@juno.com